5LMA0 – Project heat diffusion

Date:

3 February 2019

Names:			Student number:

Bob Clephas		1271431

Job Meijer		1268155

Table of contents

Introduction	3

Exercise 1	4

Exercise 2	4

Exercise 3	5

Exercise 4	6

Exercise 5	8

Exercise 6	9

Exercise 7	12

Exercise 8	12

Exercise 9	12

Exercise 10	14

# Introduction

In this project a model for hyperthermia is considered which describes how a temperature distribution on a locally heated surface evolves over time. A rectangular area is considered of dimension Lx×Ly as shown in Figure 1. Here the green areas u1 and u2 are defined as inputs where the heat fluxes are applied, the yellow area is normal tissue and the orange area is tumor tissue with dimensions lx×ly.

Figure 1 – Configuration of surface with heat flux locations

The model is described by the following parabolic partial differential equation:

ρx,ycx,yδTδtx,y,t=δδxδδyKx,yδTx,yδxδTx,yδy+u(x,y,t)

where

| Symbol | Description | Unit | Value |
| --- | --- | --- | --- |
| Lx | Length | [m] | 0.2 |
| Ly | Width | [m] | 0.15 |
| ρ | Material densities (yellow, orange area) | [kg/m3] | 1100, 1000 |
| c | Heat capacities (yellow, orange area) | [J/(kg K)] | 3890, 3350 |
| κ | Thermal conductivities (yellow, orange area) | [W/(m K)] | 0.31, 0.31 |
| (X1,Y1) | Location heat flux 1 | ([m],[m]) | (Lx/4, Ly/2) |
| (X2,Y2) | Location heat flux 2 | ([m],[m]) | (3Lx/4, Ly/2) |
| W | Actuator width | [m] | 0.05 |
| Tambient | Ambient temperature | [K] | 309 |

Table 1 – Model properties

Furthermore, the output of the model is the temperature of the surface in degrees Kelvin defined by Tambient+T(x,y,t).

The purpose of this project is to derive a fast and simple model that accurately describes the temperature evolution of this system, as explained in the following ten exercises.

# Exercise 1

In this exercise is proven whether the system is linear or nonlinear and whether the system is time-variant or time-invariant. While doing so, the two cases where the model is homogeneous or non-homogeneous are distinguished.

If the model is homogeneous (1), ρ, c and K do not depend on the position (x,y). The system is linear, because a summation of inputs results in a summation of their respective outputs (2). The system is time-invariant because the output does not change when a time shifted input is applied (3).

ρcδTδtx,y,t=κδ2Tx,y,tδx2+δ2Tx,y,tδy2+ux,y,t#1

ρcδ(T1+T2)δt=κδ2(T1+T2)δx2+δ2(T1+T2)δy2+(u1+u2)#2

ρcδTδtx,y,t+τ=κδ2Tx,y,t+τδx2+δ2Tx,y,t+τδy2+ux,y,t+τ#3

When the model is non-homogeneous (4), ρ, c and K do depend on the position (x,y). The system is still linear, because a summation of inputs still results in a summation of their respective outputs (5). The non-homogeneous system is also still time-invariant because the output does not change when a time shifted input is applied (6).

ρ(x,y)c(x,y)δTδtx,y,t=κδ2Tx,yδx2+δ2Tx,yδy2+ux,y,t#4

ρ(x,y)c(x,y)δT1+T2δt=κδ2T1+T2δx2+δ2T1+T2δy2+(u1+u2)#5

ρ(x,y)c(x,y)δTδtx,y,t+τ=κδ2Tx,y,t+τδx2+δ2Tx,y,t+τδy2+ux,y,t+τ#6

# Exercise 2

For now, it is assumed that the model is homogeneous and isotropic, meaning that lx=ly=0 (no orange area) and ρx,y=ρ, cx,y=c and Kx,y=κ are positive constants.

In this exercise it is shown that the system admits to the form Tx,y,t=atϕxϕy whenever ux,y,t=0. Here a, ϕ(x) and ϕ(y) are scalar-valued functions on R, [0,Lx] and [0,Ly], respectively, that satisfy the separated differential equations (7) for suitable constants λx, λy and λ.

ϕx-λxϕx=0,    ϕy-λyϕy=0,    a-λaa=0#7

When ux,y,t=0, this results in the following equation:

δTδt=κρcδ2Tδx2+δ2Tδy2#8

Now Tx,y,t is substituted by atϕxϕy, resulting in the following equation:

aϕ(x)ϕ(y)=κρcϕ(x)aϕ(y)+ϕ(y)aϕ(x)#9

This can be rewritten in the following three equations:

ϕx-ρcκaa-ϕyϕyϕx=0,   ϕy-ρcκaa-ϕxϕxϕy=0,   a-κρcϕxϕx+ϕyϕya=0#10

Now the following lambdas are defined:

λx=ϕxϕx,  λy=ϕyϕy,  λa=aa #11

Using these lambdas results in the following equations:

ϕx-ρcκλa-λyϕx=0,  ϕy-ρcκλa-λxϕy=0,  a-κρcλx+λya=0#12

These can be substituted which results in:

ρcκλa-λy=λx,  ρcκλa-λx=λy,  κρcλx+λy=λa#13

These lambdas are the initial conditions of the simplified model.

# Exercise 3

In this exercise is shown that for any K>0 and L>0 the set ϕk,l 0≤k≤K, 0≤l≤L} is an orthonormal set of functions in L2.

First the functions for ϕkx(x) and ϕlyy are defined:

ϕkx(x)=1Lx if k=02LxcoskπxLxif k>0		 ϕlyy=1Ly if l=02LycoslπyLy if l>0                      (14)

These functions can be combined into ϕk,lx,y, as can be seen below.

ϕk,lx,y=ϕkxxϕlyy#15

Now the inner product space L2, based on ϕk,lx,y, is computed:

ϕi,ϕj=0Lx0Lyϕix,yϕjx,ydydx#16

To proof that ϕk,l is an orthonormal set of functions in L2 the following statement has to hold:

ϕi,ϕj=1 if i=j0 if i≠j#17

The following is defined for any K>0 and L>0 and 0≤k≤K, 0≤l≤L:

ki=i mod K,  li=iK#18

The next step is to define ϕix,y, which is done for different conditions based on ki and li:

ϕix,y=1LxLy if ki=li=02LxLycosliπyLy if ki=0 and li>02LxLycoskiπxLx if ki>0 and li=02LxLycoskiπxLxcosliπyLy if ki>0 and li>0#19

When computing the inner product (16) the outcome is 1 if i=j and 0 when i≠j, leading to the conclusion that the set ϕk,l 0≤k≤K, 0≤l≤L} is an orthonormal set of functions in L2 for any K>0 and L>0.

# Exercise 4

To be able to simulate the model, The Galerkin projection is used to transform the homogeneous and isotropic form of the model (1) into an ordinary differential equation. Using the spectral decomposition of the temperature evolution (20) and the definition for the combined spectral components (15), the spectra decomposition of the temperature evolution (21) is defined.

Tx,y,t=k=0∞l=0∞ak,ltϕkxxϕlyy #20

Tx,y,t=k=0∞l=0∞ak,ltϕk,lx,y#21

Replacing the double indexing k,l with i results in the spectral decomposition of the temperature evolution (22).

Tx,y,t=i=0∞aitϕix,y #22

To calculate the Galerkin projection, the inner product of the model and the defined basis function ϕj is calculated using a standard inner product as defined in (16) which results in equation (23).

ρcδTδt,ϕj=κδ2Tδx2+δ2Tδy2+u,ϕj#23

Substituting the spectral decomposition of the temperature evolution results in equation (24)

ρci=0∞aiϕi,ϕj=κi=0∞aiϕix+i=0∞aiϕiy+u,ϕj#24

Because the defined inner product 16 is a linear operator, the equation can be rewritten into equation 25 and subsequently equation (26). From the definition it can also be seen that everything that is not a function of x or y can be treated as a constant which results in equation (27).

ρci=0∞aiϕi,ϕj= κi=0∞aiϕix,ϕj+κi=0∞aiϕiy,ϕj+u,ϕj#25

i=0∞ρcaiϕi,ϕj= i=0∞κaiϕix,ϕj+i=0∞κaiϕiy,ϕj+u,ϕj#26

i=0∞ρcaiϕi,ϕj= i=0∞κaiϕix,ϕj+ϕiy,ϕj+u,ϕj#27

Due to the orthonormality of the basis function the equation can be rewritten into equation (28).

i=0∞ρcai= i=0∞κaiϕix,ϕj+ϕiy,ϕj+u,ϕj#28

Rewriting this into matrix form for j ϵ N gives an ordinary differential equation (29).

a0⋮ai⋮a∞=κρcϕ0x,ϕ0+ϕ0y,ϕ0…ϕix,ϕ0+ϕiy,ϕ0…ϕ∞x,ϕ0+ϕ∞y,ϕ0⋮⋱⋮⋱⋮ϕ0x,ϕj+ϕ0y,ϕj…ϕix,ϕj+ϕiy,ϕj…ϕ∞x,ϕj+ϕ∞y,ϕj⋮⋱⋮⋱⋮ϕ0x,ϕ∞+ϕ0y,ϕ∞…ϕix,ϕ∞+ϕiy,ϕ∞…ϕ∞x,ϕ∞+ϕ∞y,ϕ∞a0⋮ai⋮a∞+1ρcu,ϕ0⋮u,ϕj⋮u,ϕ∞#29

Substituting the basis functions results in equation 30.

a0⋮ai⋮a∞=κρc0…0…0⋮⋱⋮⋱⋮0…-kiπLx2+liπLy2…0⋮⋱⋮⋱⋮0…0…-∞πLx2∞πLy2  a0⋮ai⋮a∞+1ρcu,ϕ0⋮u,ϕj⋮u,ϕ∞#30

To derive the equilibrium solution of the model, the input is set to 0, leading to equation 31.

a0⋮ai⋮a∞=κρc0…0…0⋮⋱⋮⋱⋮0…-kiπLx2+liπLy2…0⋮⋱⋮⋱⋮0…0…-∞πLx2∞πLy2  a0⋮ai⋮a∞#31

The free response of the model, which is in the form a=Aa, can be calculated using equation (32) resulting in equation 33.

at=eAta0#32

at=e0…0…0⋮⋱⋮⋱⋮0…e-κρckiπLx2+liπLy2t…0⋮⋱⋮⋱⋮0…0…e-κρc∞πLx2∞πLy2t  a0#33

The equilibrium solution can be obtained by equating the solution at t=∞, which results in the a coefficients as can be seen in (34).

a∞=a000⋮0#34

Filling in the coefficients (34) in equation (22) gives the equilibrium solution 35 of the model. Substituting the basis function ϕ0 and replacing a0(0) with the inner product of Initial temperature for the model and the basis function results in equation (36). Which is as expected the average temperature of the initial temperature profile.

Tx,y,∞=a00ϕ0x,y#35

Tx,y,∞=1LxLy0Lx0LyTx,y,0dydx#36

# Exercise 5

During this exercise the model is simulated using model parameters of Table 1 and no input heat fluxes. The initial temperature profile is a 2D Bell curve with height +5 [°K] and a width of 0.02 [m]. The simulation is executed with K=L=3, 5, 10, 20 as shown in Figure 2, Figure 3, Figure 4 and Figure 5.

Figure 2 – Initial temperature profile over time with K=L=3

Figure 3 – Initial temperature profile over time with K=L=5

Figure 4 – Initial temperature profile over time with K=L=10

Figure 5 – Initial temperature profile over time with K=L=20

# Exercise 6

During this exercise the simulation is executed with

- Different initial temperature profiles

- Inputs for the heat fluxes u1(t) and u2(t)

while varying with the approximation orders K and L.

The initial temperature profile is now changed to two 2D Bell curves at the location of u1(t) and u2(t) and a radius of 0.03 [m]. The Bell curve at the location of u1(t) has an initial peak temperature of +5 [°K] and the Bell curve at location u2t has an initial peak temperature of -5 [°K]. This initial profile is simulated with the same approximation orders of exercise 5. The results of these simulations are shown in Figure 6, Figure 7, Figure 8 and Figure 9.

Figure 6 – Double Bell curve initial profile with K=L=3

Figure 7 – Double Bell curve initial profile with K=L=5

Figure 8 – Double Bell curve initial profile with K=L=10

Figure 9 – Double Bell curve initial profile with K=L=20

Now the inputs u1(t) and u2(t) are added to the simulation with the following properties:

- Start at t=30

- Stop at t=330

- Heat flux = 60,000

and with the same initial temperature profile and values for K and L as in exercise 5, resulting in Figure 10, Figure 11, Figure 12 and Figure 13.

Figure 10 – Initial temperature profile with inputs u1 and u2 over time with K=L=3

Figure 11 – Initial temperature profile with inputs u1 and u2 over time with K=L=5

Figure 12 – Initial temperature profile with inputs u1 and u2 over time with K=L=10

Figure 13 – Initial temperature profile with inputs u1 and u2 over time with K=L=20

From these simulations can be concluded that with higher approximation orders for K and L the results are more accurate. If the approximation order is low, harmonics are visible in the temperature profile. With an approximation order of 3 or lower the result is inaccurate.

# Exercise 7

In this exercise the output of the simulation is taken with a high approximation order K=L=50 and this data is used to compute a POD basis ϕii=1,…,R in L2 of order R. The POD basis is created using the (economy-size) Singular Value Decomposition (SVD) of the high order approximation data Tx,y,t as shown in (37). The basis is now defined by the columns of U, U=(ϕ1,…, ϕR)

U,S,Y=svdTx,y,t,'econ'#37

# Exercise 8

Now the POD basis ϕi from exercise 7 is used to create a reduced order POD model that takes the heat fluxes u1(t) and u2(t) as inputs and outputs the coefficients ai(t) in the truncated expansion:

TRx,y,t=i=1Raitϕix,y#38

# Exercise 9

The next step is to validate the quality of the reduced order POD model from exercise 8 by simulating with different orders of R, different initial temperature profiles and different heat fluxes u1(t) and u2(t). To compare the POD basis to the earlier determined basis, the same orders for R are used as for K and L.

First the reduced order POD model is simulated with the same initial temperature profile as during exercise 5 with varying R=3,20 as shown in Figure 14 and Figure 15 which are almost identical figures.

Figure 14 – POD basis initial profile over time with order R=3

Figure 15 – POD basis initial profile over time with order R=20

Next the reduced order POD model is simulated with the same heat fluxes as during exercise 6 while varying R=3,20, as shown in Figure 16 and Figure 17 which are also almost identical.

Figure 16 – POD basis initial temperature profile with inputs u1 and u2 over time with R=3

Figure 17 – POD basis initial temperature profile with inputs u1 and u2 over time with R=20

From the figures above can be concluded that the results are almost equal, indicating that a low POD order already has a high accuracy in comparison to the earlier determined basis when the initial temperature profile and input are the same.

Now the POD basis is simulated using a different initial temperature profile. The Bell curve is moved from the position of u2(t) to the position of u1(t) as shown in Figure 18 and Figure 19.

Figure 18 - POD basis adjusted initial profile over time with order R=3

Figure 19 - POD basis adjusted initial profile over time with order R=20

Besides that, the input u1(t) and u2(t) are adjusted for the POD simulation by increasing the width of the square from 0.5 to 0.75 [m]. The results of these simulation are shown in Figure 20 and Figure 21.

Figure 20 – POD basis initial temperature profile with adjusted inputs u1 and u2 over time with R=3

Figure 21 - POD basis initial temperature profile with adjusted inputs u1 and u2 over time with R=20

From these simulations can be concluded that a reduced order POD model only results in accurate estimations if the initial conditions and inputs are unchanged.

# Exercise 10

For This exercise, a non-homogeneous material profile is considered in which a defined middle section has different ρ and c values. An ideal and approximated version of this material is visible in Figure 22.

Figure 22 - non-homogeneous material profile

The approximation of this material profile is made using a Fourier expansion according to (39), with the same basis functions as used for the homogeneous model.

ρx,ycx,y=i=0reiϕix,y#39

Now this approximation can be used to simulate the non-homogeneous model which can now be written as (40) and can be rewritten into equation (41).

i2=0rei2ϕi2i=0raiϕi,ϕj= κi=0raiϕix,ϕj+κi=0raiϕiy,ϕj+u,ϕj#40

i=0raii2=0rei2ϕi2ϕi,ϕj= i=0∞κaiϕix,ϕj+ϕiy,ϕj+u,ϕj#41

This can now be used to simulate the non-homogeneous model in the same way as the homogeneous model, only instead of dividing the right side of the equation by the constant ρc, it is now multiplied with the inverse of the approximated ρ(x,y)c(x,y) matrix. The result of the non-homogeneous model simulation is visible in Figure 23.

Figure 23 - non-homogeneous simulation

The difference with the homogeneous model is not clearly visible, the difference is that the heat does not conduct equally fast in all directions but a little faster towards the middle. The difference between the homogeneous and non-homogeneous model is better visible in the 1D case, as can be seen in Figure 24. Where the blue line represents the temperature of the non-homogeneous model and the red line the temperature of the homogeneous model.

Figure 24 - non-homogeneous simulation 1D